LAB REPORT

TP1 - Cisco Static Routing Configuration

Interconnection of two Cisco 2911 routers with static routing

| Name | ALPHA |
| --- | --- |
| First Name | Ziad Ramdan |
| Field | L1 Cybersecurity |
| School | ISM Dakar - Senegal |
| Date | March 11, 2026 |
| Tool | Cisco Packet Tracer 8.2.2 - Routeur 2911 |

# 1. Network Topology

Two Cisco 2911 routers are connected via a point-to-point link, with one PC on each side simulating two distinct local area networks. The configuration was performed from PC0 via a console cable.

| Device | Role | Connection |
| --- | --- | --- |
| PC0 | Console + LAN1 | Console cable to R1 / Straight cable Gi0/0 |
| R1 | Main Router | Gi0/0 to PC0 / Gi0/1 to R2 |
| R2 | Secondary Router | Gi0/0 to R1 / Gi0/1 to PC1 |
| PC1 | LAN2 | Straight cable to Gi0/1 of R2 |

## Logical Diagram

PC0 (192.168.1.10)

¦

[Gi0/0]

R1 LAN1: 192.168.1.0/24

[Gi0/1]

¦

10.0.0.0/30 Lien point a point

¦

[Gi0/0]

R2 LAN2: 192.168.2.0/24

[Gi0/1]

¦

PC1 (192.168.2.10)

# 2. IP Addressing Plan

| Device | Interface | IP Address | Mask | Gateway |
| --- | --- | --- | --- | --- |
| R1 | Gi0/0 | 192.168.1.1 | 255.255.255.0 /24 | - |
| R1 | Gi0/1 | 10.0.0.1 | 255.255.255.252 /30 | - |
| R2 | Gi0/0 | 10.0.0.2 | 255.255.255.252 /30 | - |
| R2 | Gi0/1 | 192.168.2.1 | 255.255.255.0 /24 | - |
| PC0 (LAN1) | Fa0 | 192.168.1.10 | 255.255.255.0 /24 | 192.168.1.1 |
| PC1 (LAN2) | Fa0 | 192.168.2.10 | 255.255.255.0 /24 | 192.168.2.1 |

Note: The /30 mask on the point-to-point link provides exactly 2 usable addresses, sufficient for a point-to-point link. Using /24 here would have wasted addresses unnecessarily.

# 3. Commands Used

## 3.1 Basic Configuration and Security

| Standard | Shortcut | Description |
| --- | --- | --- |
| enable | en | Switch to privileged mode |
| configure terminal | conf t | Enter global configuration mode |
| hostname R1 | host R1 | Set router name |
| enable secret cisco | ena sec cisco | Encrypted privileged mode password |
| line console 0 | line con 0 | Configure console access (physical) |
| password cisco1 | pass cisco1 | Set console password |
| login | login | Enable password verification |
| line vty 0 5 | line vty 0 5 | Configure remote access (6 sessions) |
| service password-encryption | ser pas | Encrypt all visible passwords |
| banner motd #message# | ban mo #message# | Display warning at login |

## 3.2 Interface Configuration

| Standard | Shortcut | Description |
| --- | --- | --- |
| interface GigabitEthernet0/0 | int gi0/0 | Enter interface configuration |
| ip address 192.168.1.1 255.255.255.0 | ip add ... | Assign IP address to interface |
| no shutdown | no sh | Enable the interface (disable shutdown) |

## 3.3 Static Routing

Without static routes, each router only knows its own networks. We must manually tell it how to reach remote networks.

| Router | Command | Meaning |
| --- | --- | --- |
| R1 | ip route 192.168.2.0 255.255.255.0 10.0.0.2 | To reach LAN2, go through R2 |
| R2 | ip route 192.168.1.0 255.255.255.0 10.0.0.1 | To reach LAN1, go through R1 |

## 3.4 Verification and Save

| Standard | Shortcut | Description |
| --- | --- | --- |
| show ip interface brief | sh ip int br | Check status of all interfaces |
| show running-config | sh ru | Display current configuration |
| show ip route | sh ip ro | Display routing table |
| copy running-config startup-config | cop ru st / wr | Save configuration |
| ping [adresse] | ping [adresse] | Test connectivity between devices |

# 4. Connectivity Test Results

| Source | Destination | Result | Note |
| --- | --- | --- | --- |
| PC0 (192.168.1.10) | R1 Gi0/0 (192.168.1.1) | 100% ✓ | Local communication OK |
| PC0 (192.168.1.10) | PC1 (192.168.2.10) | 100% ✓ | Inter-network communication OK |

Note: The first 2 packets of the ping to PC1 were lost on the first attempt. This is completely normal - the routers needed a brief moment to resolve MAC addresses via ARP. On the second attempt, success rate was 100%.

# 5. Skills Learned

- Navigate Cisco IOS modes (User, Privileged, Configuration)

- Secure a router with multiple password levels

- Encrypt passwords with service password-encryption

- Configure GigabitEthernet interfaces and enable them with no shutdown

- Build a coherent IP addressing plan with justified mask choices

- Understand and configure static routes between two networks

- Test end-to-end connectivity with the ping command

- Save configuration to non-volatile memory (NVRAM)

- Work via console cable from a PC

ALPHA Ziad Ramdan  —  L1 Cybersecurity  —  ISM Dakar  —  March 11, 2026